AI-ENABLED SMART LAMP POST FOR REAL-TIME EMERGENCY DETECTION, LOCALIZATION, AND MULTI-CHANNEL ALERTING

ABSTRACT

Safety systems deployed across densely populated urban and suburban regions predominantly operate in a response-oriented manner: an emergency usually comes to the attention of authorities only after a bystander observes it, locates a phone, and places a call. This project report presents SingaPen AI, a crisis-management system integrated directly into streetlight poles that combines physical sensing, on-device visual processing, and multiple independent wireless communication channels into a single economical unit. An MFRC522 RFID module paired with an ESP32 microcontroller allows an individual to trigger a linked identity alert, while a manual SOS button provides a parallel human-triggered pathway. Built-in redundancy is provided through a SIM800L GSM module and an SX1278 LoRa transceiver operating at 868 MHz, so that alert delivery does not depend on any single network. An onboard camera running Ultralytics YOLO11n performs edge-based weapon detection as an additional AI verification layer. All incoming data is routed to a Flask backend, with a Firebase Realtime Database collecting the live notification feed and a Leaflet-based dashboard mapping real-time incidents for dispatchers. The system accommodates two hardware forms a stationary Smart RFID Emergency Pole and a compact Portable Guardian SOS device both sharing the same alert schema and cloud backend. Campus deployment and testing at Rajalakshmi Institute of Technology, Chennai, measured sub-second end-to-end alert latency over WiFi (803 ms), reliable LoRa delivery over short range (100% packet success at 1 km, 85% at 5 km), and dependable weapon-class AI detection with a 5% false positive rate. These results indicate that integrating AI-powered emergency detection directly into existing streetlight infrastructure is an economical and effective approach to improving public safety response times.